- Prva rýchla verzia racku by mohla byt tato, a v dohladnom case ked budete mat Visio k dispozicii (treba zistit ci ho mate v baliku Office365, alebo inak...) tak nakreslit aj druhy variant priamo so zariadeniami – t.j. predny a zadny pohlad na rack tak ako ho vidime, bud pouzit ikony s visia, alebo zobrat photo z webu.

- switche a routre by bolo vhodne ocislovat, idealne zlava doprava, rack 1, rack 2, rack 3 a zariadenia v nich: switch 1.1, 1.2, … 2.1, 2.2, ...router 1.1, 1.2.

### Provisioning Rack &  HLR-FE Rack

### Power cabling

Tu by sme potrebovali nakreslit ako su zretazene  predlzovacky dole v rackoch… vzdy vrchna zapina spodnu -  2 su vzdy zretazene, treba spocitat ktory konektor je ktore zariadenie a nejako to nakreslit nazorne. T.j. porty v predlzovacke ocislovat a napisat pre ake zariadenie je to (switch, router...) - switche a routre by bolo vhodne ocislovat, idealne zlava doprava, rack 1, rack 2, rack 3 a zariadenie v nich: switch 1.1, 1.2, router 1.1, 1.2..

# Room layout

## Prague CTX

Tu by sme potrebovali takyto layout aj s rozmermi miestnosti – idealne kreslit vo Visiu a zdrojovy document (visio) s nakresom dat zdielat na OneDrive tiez a aj tu vlozit obrazok.

# Site Prague CTX

## HLRd Core Components... RB301 Rack 01

- Do popisu pre zariadenia:

  - Model zariadenia

  - Verzia IOSu

  - Pamäte

  - Procesor

  - Porty

  - Čo ešte iné?

| Hardware component |  | Characteristics | Role | Quantity on site |
| --- | --- | --- | --- | --- |
|  | SUN Netra T2000 | -8xUltraSPARC64 CPU @ 1.2 GHz -32GB DDR2 Ram -2x146GB Disk capacity -3 PCI-X, 1 PCI-E slots, -4 onboard 10/100/1000 Ethernet ports -1 Quad Gigabit Ethernet PCI card | Used as HLR Front End | 3 |
|  | Sun Netra v210 | -1xUltraSPARC Illi CPU @ 1.3 GHz -1GB SDRAM -1x73GB Disk capacity -4 onboard 10/100/1000 Ethernet ports | Used as Install Server for HLR Front End | 1 |
|  | Utimaco SafeGuard Crypto Server CS10 | -Supports all commonly available symmetrical and asymmetrical cryptographic procedures and protocols -Can generate up to 100 signatures per second | Hardware Security Module (HSM) | 2 |
|  | FSC Primergy RX600-S4 | -3xIntel Xeon Dual Core Processor (3.0 GHz, 667 MHz FSB) -16GB Ram -2x500GB Disk capacity -2 10/100/1000 MBit/s Ethernet boards | Used as HLRd Database Front End | 2 |
|  |  |  | Used as Provisioning Gateway Directory Server Agent (PGW-DSA) | 1 |
|  |  |  | Used as Provisioning Gateway (PGW) | 1 |
|  |  | -2xIntel Xeon Dual Core Processor (3.0 GHz, 667 MHz FSB) -32GB Ram -2x500GB Disk capacity -2 10/100/1000 MBit/s Ethernet boards | Used for Database Management | 1 |
|  |  |  | Serves as HLRd Database Back End | 4 |
|  |  |  | Serves as Install Server for C-NTDB | 1 |
|  |  |  | Used as OneNDS DB Manager | 1 |
|  | FSC Primergy RX300 S3 | -2x Intel Xeoon dual core 1,6 GHz -4GB Ram -2X73 GB Disk capacity | Used as CITRIX Server for remote access | 1 |

## Network equipment (provided by TMCZ)

| Hardware component |  | Characteristics | Role | Quantity on site |
| --- | --- | --- | --- | --- |
|  | Cisco Catalyst 2960G-24TC-L | - 24x1Gbps | Used as PGW Switch | 2 |
|  |  |  | Used as HSM Switch | 2 |
|  | Cisco Catalyst 4948-E | -48x1Gbps -redundant fans -redundant hot plug power supplies | Used as LAN Switch | 4 |
|  | Cisco ASA 5580 | - Two USB ports - Two RJ-45 Management ports - Two Gigabit Ethernet Management ports With Interface Expansion Cards: - Up to Twelve 10Gigabit Ethernet (10GE) ports - Up to Twenty-Four Gigabit Ethernet ports - Up to Twenty-Four 10/100/1000 Ethernet ports | Used as Firewall | 2 |
|  | Cisco Catalyst 6506 | - Supervisor engines - Fast Ethernet modules (with IEEE 802.3af Power over Ethernet [PoE]) - Gigabit Ethernet modules (with IEEE 802.3af PoE) - 10 Gigabit Ethernet modules - Flex WAN modules | Used as CE router | 2 |

## Additional components

| Hardware component |  | Characteristics | Role | Quantity on site |
| --- | --- | --- | --- | --- |
|  | Primecenter Rack 38 RU | - 38U rack (H x W x D) 1825 x 700 x 1000 mm Transport dimensions: 1935 x 775 x 1180 mm | Host all HLRd solution components | 4 |
|  | Primergy RC23 Rack console | -17-inch TFT flat screen display -Connection for PS/2 and USB -Telescopic rails with variable depth from 704 to 790 mm -Integrated cable management - German/International keyboard layout | Console of the HLRd Primecenter Racks | 4 |
|  | Primepower Console Switches | -4/8 and 16 port switch -End-to-end CAT5 connections -GUI client for remote access -Multi-platform switch | Console switch for HLRd Primecenter Racks | 4 |
|  | Triton DeltaS 800x900 rack : | - 45U rack Dimensions (H x W x D) 2105 x 687 x 900 mm | Used to host network components (Provided by TMCZ) | 2 |
|  | Big IP F5 LTM-1500 | - high availability of IP applications/services running across multiple data centers -local area application traffic management solution - scalable SSL processing platform | Used for inter-connection and load-balancing between customer’s provisioning system and HLRd’s Provisioning Gateway (PGW) | 2 |

# Site Brno

| Hardware component |  | Characteristics | Role | Quantity on site |
| --- | --- | --- | --- | --- |
|  | SUN Netra T2000 | -8xUltraSPARC64 CPU @ 1.2 GHz -32GB DDR2 Ram -2x146GB Disk capacity -3 PCI-X, 1 PCI-E slots, -4 onboard 10/100/1000 Ethernet ports -1 Quad Gigabit Ethernet PCI card | Used as HLR Front End | 3 |
|  | Sun Netra v210 | -1xUltraSPARC Illi CPU @ 1.3 GHz -1GB SDRAM -1x73GB Disk capacity -4 onboard 10/100/1000 Ethernet ports | Used as Install Server for HLR Front End | 1 |
|  | Utimaco SafeGuard Crypto Server CS10 | -Supports all commonly available symmetrical and asymmetrical cryptographic procedures and protocols -Can generate up to 100 signatures per second | Hardware Security Module (HSM) | 2 |
|  | FSC Primergy RX600-S4 | -3xIntel Xeon Dual Core Processor (3.0 GHz, 667 MHz FSB) -16GB Ram -2x500GB Disk capacity -2 10/100/1000 MBit/s Ethernet boards | Used as HLRd Database Front End | 2 |
|  |  |  | Used as Provisioning Gateway Directory Server Agent (PGW-DSA) | 1 |
|  |  |  | Used as Provisioning Gateway (PGW) | 1 |
|  |  | -2xIntel Xeon Dual Core Processor (3.0 GHz, 667 MHz FSB) -32GB Ram -2x500GB Disk capacity -2 10/100/1000 MBit/s Ethernet boards | Used for Database Management | 1 |
|  |  |  | Serves as HLRd Database Back End | 4 |
|  |  |  | Serves as Install Server for C-NTDB | 1 |
|  |  |  | Used as OneNDS DB Manager | 1 |

## HLRd Core Components

## Network equipment (provided by TMCZ)

| Hardware component |  | Characteristics | Role | Quantity on site |
| --- | --- | --- | --- | --- |
|  | Cisco Catalyst 2960G-24TC-L | - 24x1Gbps | Used as PGW Switch | 2 |
|  |  |  | Used as HSM Switch | 2 |
|  | Cisco Catalyst 4948-E | -48x1Gbps -redundant fans -redundant hot plug power supplies | Used as LAN Switch | 4 |
|  | Cisco ASA 5580 | - Two USB ports - Two RJ-45 Management ports - Two Gigabit Ethernet Management ports With Interface Expansion Cards: - Up to Twelve 10Gigabit Ethernet (10GE) ports - Up to Twenty-Four Gigabit Ethernet ports - Up to Twenty-Four 10/100/1000 Ethernet ports | Used as Firewall | 2 |
|  | Cisco Catalyst 6506 | - Supervisor engines - Fast Ethernet modules (with IEEE 802.3af Power over Ethernet [PoE]) - Gigabit Ethernet modules (with IEEE 802.3af PoE) - 10 Gigabit Ethernet modules - Flex WAN modules | Used as CE router | 2 |

## Additional components

| Hardware component |  | Characteristics | Role | Quantity on site |
| --- | --- | --- | --- | --- |
|  | Primecenter Rack 38 RU | - 38U rack (H x W x D) 1825 x 700 x 1000 mm Transport dimensions: 1935 x 775 x 1180 mm | Host all HLRd solution components | 4 |
|  | Primergy RC23 Rack console | -17-inch TFT flat screen display -Connection for PS/2 and USB -Telescopic rails with variable depth from 704 to 790 mm -Integrated cable management - German/International keyboard layout | Console of the HLRd Primecenter Racks | 4 |
|  | Primepower Console Switches | -4/8 and 16 port switch -End-to-end CAT5 connections -GUI client for remote access -Multi-platform switch | Console switch for HLRd Primecenter Racks | 4 |
|  | Triton DeltaS 800x900 rack : | - 45U rack Dimensions (H x W x D) 2105 x 687 x 900 mm | Used to host network components (Provided by TMCZ) | 2 |